MONITOR

PROGRAM

SOURCE LISTING

COPYRIGHT

Copyright ©1981 by MULTITECH INDUSTRIAL CORP. All rights reserved. No part of this publication may be, reproduced, transmitted, transcribed, stored in a retrieval system, or translated into any language or computer language, in any form or by any means, electronic, mechanical, magnetic, optical, chemical, manual or otherwise, without the prior written permission of MULTITECH INDUSTRIAL CORP.

DISCLAIMER

MULTITECH INDUSTRIAL CORP. makes no representations or warranties, either express or implied, with respect to the contents hereof and specifically disclaims any warranties or merchantability or fitness for any particular purpose. MULTITECH INDUSTRIAL CORP. software described in this manual is sold or licensed "as is". Should the programs prove defective following their purchase, the buyer (and not MULTITECH INDUSTRIAL CORP., its distributor, or its dealer) assumes the entire cost of all necessary servicing, repair, and any incidental or consequential damages resulting from any defect in the software. Further, MULTITECH INDUSTRIAL CORP. reserves the right to revise this publication and to make changes from time to time in the content hereof without obligation of MULTI-TECH INDUSTRIAL CORP. to notify any person of such revision or changes.

|  | /Multitech INDUSTRIAL CORP. OFFICE/ 9FL, 266 SUNG CHIANG ROAD, TAIPEI 104' TAIWAN R.O.C. TEL:(02)551-1101 TELEX:"19162 MULTIIC" FAX:(02) 542-2805 FACTORY/ 1 INDUSTRIAL E. RD., III HSINCHU SCIENCE - BASED INDUSTRIAL PARK, HSINCHU, TAIWAN, R.O.C. |
| --- | --- |

1

MPF-I

LOC OBJ CODE M STMT SOURCE STATEMENT

1 ;***********************************************************

2 ;*

3 ;*	COPYRIGHT , MULTITECH INDUSTRIAL CORP. 1981

4 •*	All right reserved.

5 ;*	No part of this software may be copied without

6 ;*	the express written consent of MULTITECH

7 ;*	INDUSTRIAL CORP.

8	•*

9 .***********************************************************

10

11	;

12

13

14

15 P8255	EQU	03H	;8255 I control port

16 DIGIT	EQU	02H	;8255 I port C

17 SEG7	EQU	01H	;8255 I port B

18 KIN	EQU	OOH	;8255 I port A

19 PWCODE EQU	GASH	;Power-up code

20 ZSUM	EQU	71H	;This will make the sun of all

21	;monitor codes to be zero.

22

23 ; The following EQUATEs are used for timing. Their values

24 ; depend on the CPU clock frequency. (In this version, the

25 ; crystal frequency is 1.79 MHz.)

26

27 COLDEL EQU	201	;Column delay time for routine

28	;SCAN and SCAN1.

29 F1KHZ	EQU	65	;Delay count for 1K Hz square wave,

30	;used by routin TONE1K.

31 F2KHZ	EQU	31	;Delay count for 2K Hz square wave,

32	;used by routine TONE2K.

33 MPERIOD EQU	42	;1K Hz and 2K Hz threshold, used by

34	;tape input routine PERIOD.

35

36 ; The following EQUATEs are for tape modulation.

37 ; If the quality of tape recorder is good, the user may

38 ; change '4 4 2 8' to '2 2 1	This will double

39 ; the tape data rate.

40 ; If the quality of tape recorder is poor, the user may

41 ; change '4 4 2 8 ' to '6 6 3 12'. This will improve

42 ; error performance but slow down the data rate.

43 ; Although the data format is changed, the tape is still

44 ; compatible in each case, because only the ratio is,

45 ; detected in the Tape-read.

46

47 ONE 1K EQU	4

48 ONE 2K EQU	4

49 ZERO 1K EQU	2

50 ZERO 2K EQU	8

51

52 .***********************************************************

53 ; I/O port assignment: (8255 I)

54

55 ; port A (address 00H):

56 ;	bit 7 -- tape input

57 ;	bit 6 -- 'USER KEY' on keyboard, active low

58 ;	bit 5-0 row of keyboard matrix input ,active low

2

| LOC | MPF-I OBJ CODE M STMT SOURCE STATEMENT 59 ; port B (address 01H): 7 segaments of LED, active high |  |  |
| --- | --- | --- | --- |
|  |  | 60 | ; bit 7 -- segament d |
|  |  | 61 | ; bit 6 -- decimal point |
|  |  | 62 | ; bit 5 -- segament c |
|  |  | 63 | ; bit 4 -- segament b |
|  |  | 64 | ; bit 3 -- segament a |
|  |  | 65 | ; bit 2 -- segament f |
|  |  | 66 | ; bit 1 segament g |
|  |  | 67 | ; bit 0 segament e |
|  |  | 68 | ; port C (address 02H): |
|  |  | 69 | ; bit 7 -- tape & tone output |
|  |  | 70 | ; bit 6 -- BREAK enable. NMI (CPU pin 17) will goes to |
|  |  | 71 | ; low 5 Ml's (machine cycle one) after this |
|  |  | 72 | ; bit goes to low. (This bit is connected to |
|  |  | 73 | ; the reset input of external counter.) |
|  |  | 74 | ; bit 500 -- columns of keyboard and display matrix, |
|  |  | 75 | ; active high. Bit 5 is the leftmost column. |
|  |  | 76 |  |
|  |  | 77 | •*********************************************************** |
|  |  | 78 | ; -- reset -- |
|  |  | 79 | ; There are two cases that will generate a RESET signal: |
|  |  | 80 | ; (i) power-up |
|  |  | 81 | ; (ii) 'RS' key pressed |
|  |  | 82 | ; In both cases, the follow actions will be taken: |
|  |  | 83 | ; a) disable interrupt, set interrupt mode to 0 |
|  |  | 84 | ; set I register to 00 and start execution |
|  |  | 85 | ; at address 0000 (by Z80 CPU itself). |
|  |  | 86 | ; b) initial user's PC to the lowest RAM address; |
|  |  | 87 | ; c) set user's SP to 1F9FH; |
|  |  | 88 | ; d) set user's I register to 00 and disable user's |
|  |  | 89 | ; interrupt flip-flop; |
|  |  | 90 | ; In addition, subroutine INI will be called on power-up |
|  |  | 91 | ; reset, which has the following effects: |
|  |  | 92 | ; e) disable BREAK POINT; |
|  |  | 93 | ; f) set the contents Of location 1FEEH 1FEFH to 66 and |
|  |  | 94 | ; and 00 respectively. This will make instruction RST |
|  |  | 95 | ; 38H (opcode FF) have the same effect as BREAK. |
|  |  | 96 | ; Memory location POWERUP is used to distinguish power-up |
|  |  | 97 | ; from RS-key. (POWERUP) contains a random data when |
|  |  | 98 | ; power-up and contains PWCODE (0A5H) thereafter. |
|  |  | 99 |  |
| 0000 | 0600 | 100 | LD B4O |
| 0002 | 10FE | 101 | DJNZ $ ;Power-up delay |
|  |  | 102 |  |
|  |  | 103 | ;Initial 8255 to mode 0 with port A input, port B and C |
|  |  | 104 | ; output. The control word is 90H: |
|  |  | 105 |  |
| 0004 | 3E90 | 106 | LD A,10010000B |
| 0006 | D303 | 107 | OUT (P8255),A |
|  |  | 108 |  |
|  |  | 109 | ; When the control word is sent out to 8255, all output |
|  |  | 110 | ; ports are cleared to O. It is necessary to disable |
|  |  | 111 | ; BREAK and deactivate all I/O by send(ng OCOR to |
|  |  | 112 | ; port C. |
|  |  | 113 |  |
| 0008 | 3ECO | 114 | LD A,OCOH |
| 000A | D302 | 115 | OUT (DIGIT),A |
| 000C | 31AF1F | 116 | LD SP,SYSSTK ;initial system stack |

3

| LOC | MPF —I OBJ CODE M STMT SOURCE STATEMENT 117 118 ; If the content of location POWERUP is not equal to |  |  |
| --- | --- | --- | --- |
|  |  | 119 | ; PWCODE, call subroutine 1141. Continue otherwise. |
|  |  | 120 |  |
| 000F | 3AE51F | 121 | LD A,(POWERUP) |
| 0012 | FEA5 | 122 | CP PWCODE |
| 0014 | C4C103 | 123 | CALL NE, INI |
|  |  | 124 |  |
|  |  | 125 | ; Determine the lowest RAM address by checking whether |
|  |  | 126 | ; address 1000H is RAM. If yes, set user's PC to this |
|  |  | 127 | ; value. Otherwise, set it to 1800H. |
|  |  | 128 |  |
| 0017 | 210010 | 129 | LD HL,1000H |
| 001A | CDF605 | 130 | CALL RAMCHK |
| 001D | 2802 | 131 | JR Z,PREPC |
| 001F | 2618 | 132 | LD H,18H |
| 0021 | 22DC1F | 133 | PREPC LD (USERPC),HL |
| 0024 | 2600 | 134 | LD H,0 |
|  |  | 135 |  |
|  |  | 136 | ; Address 28H and 30H are reserved for BREAK (RST 28H) |
|  |  | 137 | ; and software BREAK (RST 30H). Skip these area, monitor |
|  |  | 138 | ; program resumes at RESET1. |
|  |  | 139 |  |
| 0026 | 180A | 140 | JR RESET1 |
|  |  | 141 |  |
|  |  | 142 | .******************************************** *********** |
| 0028 |  | 143 | RST28 ORG 28H |
|  |  | 144 | ; Address 28H is the entry point of BREAK trap. |
|  |  | 145 | ; If a location is set as a BREAK point, the monitor |
|  |  | 146 | ; will change the content of this location to C7 (RST 28H) |
|  |  | 147 | ; before transfering control to user's program. |
|  |  | 148 | ; In execution of user's program, a trap will occur if |
|  |  | 149 | ; user's PC passes this location. The monitor then takes |
|  |  | 150 | ; over control and the content of BREAK address |
|  |  | 151 | ; will be restored. Monitor takes care of everything |
|  |  | 152 | ; and makes the whole mechanism transparant to the user. |
|  |  | 153 | ; The return address pushed onto stack is the PC after |
|  |  | 154 | ; executing RST 28H. The original break address should |
|  |  | 155 | ; be one less than that. The following 3 instructions |
|  |  | 156 | ; decrease the content of (SP) by one without changing |
|  |  | 157 | ; HL. |
|  |  | 158 |  |
| 0028 | E3 | 159 | EX (SP),HL |
| %0029 | 2B | 160 | DEC HL |
| 002A | E3 | 161 | EX (SP),HL |
| 002B | 22E81F | 162 | LD (HLTEMP),HL |
| 002E | 180E | 163 | JR CONT28 |
|  |  | 164 | ; |
|  |  | 165 | •*********************************************************** |
| 0030 |  | 166 | RST30 ORG 30H |
|  |  | 167 |  |
|  |  | 168 | ; Instruction RST 30H (opcode F7) is usually used as: |
|  |  | 169 | ; i) Software break; |
|  |  | 170 | ; ii) Terminator ofuser's program. |
|  |  | 171 | ; The effect of this instruction is to save all user's |
|  |  | 172 | ; registers and return to monitor. |
|  |  | 173 |  |
| 0030 | 1834 | 174 | JR NMI |